R8245 _Sapphire_Locomotive Decoder

The Sapphire is decoder is NMRA compliant and tested. It also supports RailCom® and asymmetric DCC.

The Sapphire is fully compatible with both the Select and Elite systems.

The points below outline the Sapphires main benefits:

Built to conform to all relevant NMRA Standards.

High frequency back-EMF motor control - high frequency to reduce audible noise.

Supports RailCom®.

Asymmetrical DCC support.

Size: Length 23.0 mm x Width 17.0 mm x Height 5.0 mm.

Low speed gear for shunting operations.

14,28 and 128 speed steps.

Optional operation on conventional DC layouts.

Provides 1A continuous motor current, 1.5A stall.

Four function outputs rated at 200mA each.

Total current sum of function outputs = 500mA.

Directional or independent lighting with dimming and extensive special effects.

Supports advanced consist control and extended addressing.

Support for programming on the mainline (operations mode programming).

Support for all form of programming as described in NMRA RP-9.2.3.

21-pin connector.

Important Note:

The locomotive must be removed from the track before installing or removing a locomotive decoder.

To prevent the risk of electro-static damage occurring to the decoder, NEVER directly touch the component parts of a decoder PCB.

Detailed specification:

1.1 General specification

| Function | Description |
| --- | --- |
| NMRA standards | Designed and built to NMRA standards: S 9.1, S 9.2, RP 9.1.1, RP 9.2.1,RP 9.2.2, RP 9.2.3, RP 9.2.4, RP 9.3.1, RP 9.3.2. |
| NMRA Plug | 21-pin system. |
| Current capacity | Total max continuous current carrying capacity: 1.5A. Motor output: 1A continuous, 1.5A stall. Functional output 1,2,3,4:200mA each. Total current-carrying capacity of function outputs: 500mA. |
| Protection | Motor overload protection. Functional output overload protection. |
| Motor drive | HEX FET H-Bridge drive for brushed DC motors. High-frequency control to reduce the audible noise. It is recognised that different motors require different feedback control algorithms and hence it is possible to select the algorithm. It is possible to fine tune performance via the various CV values. The algorithms will not support brushless or AC motors. The minimum, maximum and mid speed can be set and the decoder adapts the characteristic speed line dynamically to ensure a steady, even curve. It is also possible to program an individual characteristic speed curve. |
| Speed steps | 14,28,128. |
| Size | Length 23.0mm x Width 17.0 mm x Height 5.0mm. |
| Addressing | Standard (basic) and advanced consist control and extended addressing. |
| Programming | Support for programming on the mainline (operations mode programming). Support for all form of programming as described in NMRA RP-9.2.3. |
| Starting & braking delay | Starting and breaking delays can be set independently of each other. |
| Shunting speed | A shunting speed mode can be entered which halves the speed of the locomotive and hence prevents accidents from happening. |
| Simulation | Amounts of water and fuel can be stored in the decoder which will enable simulation driving. Burn rate is proportional to the current speed the locomotive is travelling as normalized to a 128 speed step value. |

| Function | Description |
| --- | --- |
| Asymmetrical DCC | The decoder is compatible with asymmetrical DCC and hence can automatically respond to certain sections of the layout and provide autonomous operation. |
| "Auto" control | Locomotive can start/stop based upon timer junctions, without input from the controller. |
| Clock | Built-in clock — this will be re-set when the power is cycled. |
| RailCom® | The decoder has complete bi-directional communication features. |
| Function outputs | The decoder has four function outputs can be mapped to the functions of the digital system according to NMRA specifications. |
| Lighting effects | Different lighting effects can be set at the four function outputs: • steady state, • strobe, • flickering/random fast flash, • flashing. |

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
| Primary Address | 1 | 1-127 | 3 | Bits 0-6 contain an address with a value between 1 and 127. Bit seven must have a value of 0". If the value of Configuration Variable # 1 is "00000000" then the decoder will go out of NMRA digital mode and convert to the alternate power source as defined by Configuration Variable #12. |
| Vstart | 2 | 0-255 | 0 | Vstart is used to define the voltage drive level used as the start voltage on the motor. |
| Acceleration Rate | 3 | 0-255 | 5 | Starting. |
| Deceleration Rate | 4 | 0-255 | 5 | Braking Delay. |
| Vhigh | 5 | 0-255 | 255 | Vhigh is used to specify the motor voltage drive levels at the maximum speed step. |
| Vmid | 6 | 0-255 | 48 | Vmid specifies the voltage drive level at the middle speed step. |
| Manufacturer Version No. | 7 | 10 | 10 | Manufacturer defined version info. |

table continues

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
| Manufactured ID | 8 | 48 | 48 | Values assigned by NMRA. |
| EMF | 10 | 1-128 | 128 | Contains a value between 1 and 128 that indicates the speed |
| Feedback Cutout |  |  |  | step above which the back EMF motor control cuts off. |
| Extended | 17 | 192-231 | 192 | The Extended Address is the locomotives address when the decoder |
| Address High- |  |  |  | is set up for extended addressing (indicated by a value of "1" in |
| Order Byte |  |  |  | bit location 5 of CV#29). CV#17 contains the most significant |
|  |  |  |  | bits of the two byte address and must have a value between |
| Extended | 18 | 0-255 | 100 | 11000000 and 11100111, inclusive, in order for this two byte |
| Address Low- |  |  |  | address to be valid. CV 18 contains the least significant bits of |
| Order Byte |  |  |  | the address and may contain any value. |
| Consist | 19 | 0-255 | 0 | Contains a seven bit address in bit positions 0-6. |
| Address |  |  |  | Bit 7 indicates the relative direction of this unit within a consist. |
| Bi-Directional Communication Configuration - RailCom® | 28 | 0-7 | 7 | Used to Configure decoder's Bi-Directional communication characteristics when CV29-Bit 3 is set. Bit RailCom® configuration 7(dec). Bit 0: channel 1 release for address broadcast 1. Bit 1: channel 2 release for data 1. Bit 2: channel 1 release for command acknowledge 1. |

table continues

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
| Configuration Data #1 | 29 | 0-48 | 6 | Configuration byte. Bit 0 = Locomotive Direction: "0" = normal," 1" = reversed. This bit controls the locomotive’s forward and backward direction in digital mode only. Directional sensitive functions, such as headlights (FL and FR), will also be reversed so that they line up with the locomotive's new forward direction. Bit 1 = FL location:"0"= bit 4 in Speed and Direction instructions Bit 2 = Power Source Conversion:"0" = NMRA DigitalOnly,"1" = Power Source Conversion Enabled. Bit 3 = Bi-Directional Communications: “0" = Bi-Directional Communications disabled, "1" = Bi-Directional Communications enabled. Bit 4 = Speed Table: "0" speed table set by configuration variables #2, #5, and #6, "1"= Speed Table set by configuration variables #66-#95. Bit 5 = "0"= one byte addressing,"1" = two byte addressing (also known as extended addressing). Bit 6 = Reserved for future use. Bit 7 = Accessory Decoder: "0" = Multifunction Decoder,"1" = Accessory Decoder (see CV #541 for a description of assignments for bits 0-6). |

table continues

| CV Name | CV | Range | Default | Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ERROR Information | 30 | 0-7 | 0 | B7 | B6 | B4 | B5 | B3 | B2 | B1 | B0 |
|  |  |  |  | X | X | X | X | X | E3 | E2 | E1 |
|  |  |  |  | Bit 0 Motor short-circuit. Bit 1 Light (OUTPUT 0-1) short-circuit. Bit 2 Light (OUTPUT 2-3) short-circuit. Bit 3-7 Always 0. |  |  |  |  |  |  |  |
| Function Mapping | 33-36 |  |  | Contains a matrix of which function inputs control which outputs. |  |  |  |  |  |  |  |

|  |  |  | Default Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Binary |  |  |  |  |  |  |  |
| CV Name | CV | Range | Decimal | Bit7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 (Light 3) | Bit 2 (Light 2) | Bit 1 (Light 1) | Bit 0 (Light 0) |
| Light activated upon F0 Forward | 33 | 0-15 | 1 | X | X | X | X | 0 | 0 | 0 | 1 |
| Light activated upon F0 Reverse | 34 | 0-15 | 2 | X | X | X | X | 0 | 0 | 1 | 0 |
| Light activated upon F1 | 35 | 0-15 | 4 | X | X | X | X | 0 | 1 | 0 | 0 |
| Light activated upon F2 | 36 | 0-15 | 8 | X | X | X | X | 1 | 0 | 0 | 0 |

| X : unimplemented, always 0. Value in bit 3-0 of CV33-36: |  | 1 - activate the corresponding light, 0 - deactivate the corresponding light. |  |  |
| --- | --- | --- | --- | --- |
| Read/Set clock - Hours | 50 | 0-23 | 0 | Real Time Clock: hours, Only Readable/writeable in Operational Mode |
| Read/Set do - Mins | 51 | 0-59 | 0 | Real Time Clock: mins, Only Readable/writeable in Operational Mode. |
| CV Name; CV; Default; Description. Asymmetrical DCC Configuration; 52; 0; See table. Value in CV52; ADCC Function; Bit 2 (Bit 3); Bit 1 (Bit 2); Bit 0 (Bit 1). 0; ADCC operation turned OFF; 0; 0; 0. 1; Loco is only able to enter ADCC mode with the correct travelling direction. Loco comes to standstill when it detects the ADCC signal. Users are able to manually drive the loco i.e. to reverse away from the ADCC signal.; 0; 0; 1. 2; Not applicable. 3; Loco is able to enter ADCC mode from either direction. Loco comes to standstill when it detects ADCC signal. Users are able to manually drive the loco i.e. reverse away from the ADCC signal.; 0; 1; 1. 4; Not applicable. 5; Loco is only able to enter ADCC mode with the correct travelling direction Loco comes to an intermediate stop when it detects the ADCC signal. The loco will then resume traveling in the same direction after the intermediate stopping time(defined in CV53) has expired.; 1; 0; 1. 6; Not applicable. 7; Loco is able to enter ADCC mode from either direction. Loco comes to an intermediate stop when it detects the ADCC signal. The loco will then resume traveling in the same direction after the intermediate stopping time(defined in CV53) has expired.; 1; 1; 1. |
| Intermediate stopping time | 53 | 0-255 | 3 | Intermediate stopping time in ADCC (in Sec). |

| CV Name | CV | Range | Description |
| --- | --- | --- | --- |
| Speed Table | 67-94 | 0-255 | Speed table. |
|  | 67 | 0-255 | 1 |
|  | 68 | 0-255 | 2 |
|  | 69 | 0-255 | 3 |
|  | 70 | 0-255 | 5 |
|  | 71 | 0-255 | 8 |
|  | 72 | 0-255 | 11 |
|  | 73 | 0-255 | 16 |
|  | 74 | 0-255 | 20 |
|  | 75 | 0-255 | 26 |
|  | 76 | 0-255 | 31 |
|  | 77 | 0-255 | 39 |
|  | 78 | 0-255 | 45 |
|  | 79 | 0-255 | 54 |
|  | 80 | 0-255 | 62 |
|  | 81 | 0-255 | 72 |
|  | 82 | 0-255 | 81 |
|  | 83 | 0-255 | 92 |
|  | 84 | 0-255 | 102 |
|  | 85 | 0-255 | 115 |
|  | 86 | 0-255 | 126 |
|  | 87 | 0-255 | 141 |
|  | 88 | 0-255 | 153 |
|  | 89 | 0-255 | 169 |
|  | 90 | 0-255 | 182 |
|  | 91 | 0-255 | 199 |

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
|  | 92 | 0-255 | 217 |  |
|  | 93 | 0-255 | 232 |  |
|  | 94 | 0-255 | 255 |  |
| User Identifier number #1 | 105 | 0-255 | 255 | For customer use. |
| User Identifier number #2 | 106 | 0-255 | 255 | For customer use. |
| Signal Time Out | 111 | 0-59 | 20 | Signal Time Out( second). While in digital operations mode each Multi Function Digital Decoder shall have a Packet Update time out value. While in digital operations mode, if the packet time-out value is exceeded, the Multi Function Digital Decoder will bring to a stop all controlled devices. The purpose of this time-out is to insure that each Multi Function Digital Decoder receives a periodic update from the Digital Command Station and thereby help prevent runaway conditions. The user should be able to define the value for this time-out within these restrictions: - a value of 0 disables the time-out (i.e., the user has chosen not to have a time-out). - a value range of 1 through TIMEOUT_MAX sets the time-out to the chosen value. The minimum value of TIMEOUT_MAX will be 20 seconds. It may be longer at the manufacturer’s discretion. |
| Shunting mode | 112 | 0-1 | 0 | 0- Shunting Mode Off. |
|  |  |  |  | 1- Shunting Mode On. |
| Light effect of function output 0 | 113 | 0-3 | 0 |  |
|  |  |  |  | 0 - steady state. |
|  |  |  |  | 1 - strobe. |
| Light effect of function output 1 | 114 | 0-3 | 0 | 2 - flickering / random fast flash. |
|  |  |  |  | 3 - steady flashing. |

| CV Name | CV | Range | Default | Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Light effect of function output 2 | 115 | 0-3 | 0 |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 - steady state. |  |  |  |  |  |  |  |
|  |  |  |  | 1 - strobe. |  |  |  |  |  |  |  |
| Light effect of function output 3 | 116 | 0-3 | 0 | 2 - flickering / random fast flash. |  |  |  |  |  |  |  |
|  |  |  |  | 3 - steady flashing. |  |  |  |  |  |  |  |
| Dimming Value for Output 0 | 117 | 0-15 | 15 | Sets brightness for Output 0 for Light effect - steady state. |  |  |  |  |  |  |  |
| Dimming Value for Output 1 | 118 | 0-15 | 15 | Sets brightness for Output 1 for Light effect - steady state. |  |  |  |  |  |  |  |
| Dimming Value for Output 2 | 119 | 0-15 | 15 | Sets brightness for Output 2 for Light effect - steady state. |  |  |  |  |  |  |  |
| Dimming Value for Output 3 | 120 | 0-15 | 15 | Sets brightness for Output 3 for Light effect - steady state. |  |  |  |  |  |  |  |
| Delay time -hour, before Events start | 121 | 0-23 | 0 | Set delay time : hour. |  |  |  |  |  |  |  |
| Delay time -min, before Events start | 122 | 0-59 | 0 | Set delay time : min. |  |  |  |  |  |  |  |
| Delay time -sec, before Events start | 123 | 0-59 | 0 | Set delay time : sec. |  |  |  |  |  |  |  |
| Event 1 -action | 124 | 0-255 | 0 | Set the speed and direction of locomotive in vent 1. |  |  |  |  |  |  |  |
|  |  |  |  | CV 124,128,132,136 action specification bit 0 to bit 6 (S0-S6) determine the normalized Speed (0 - 126) bit 7(Dir) determines Direction, normalized. By CV29 - default dir bit. |  |  |  |  |  |  |  |
|  |  |  |  | B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
|  |  |  |  | Dir | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
| Event 1 acting time - hour | 125 | 0-23 | 0 | Set event 1 running time : hour. |  |  |  |  |  |  |  |
| Event 1 acting time - min | 126 | 0-59 | 0 | et event 1 running time : min. |  |  |  |  |  |  |  |

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
| Event 1 acting time - sec | 127 | 0-59 | 0 | Set event 1running time : sec. |
| Event 2 -action | 128 | 0-255 | 0 | Set the speed and direction of locomotive in event 2. |
| Event 2 acting time - hour | 129 | 0-23 | 0 | Set event 2 running time : hour. |
| Event 2 acting time - min | 130 | 0-59 | 0 | Set event 2 running time : min. |
| Event 2 acting | 131 | 0-59 | 0 | Set event 2running time : sec. |
| Event 3 - action | 132 | 0-255 | 0 | Set the speed and direction of locomotive in event 3. |
| Event 3 acting time - hour | 133 | 0-23 | 0 | Set event 3 running time: hour. |
| Event 3 acting time - min | 134 | 0-59 | 0 | Set event 3 running time : min. |
| Event 3 acting | 135 | 0-59 | 0 | Set event 3 running time : sec. |
| Event 4 action | 136 | 0-255 | 0 | Set the speed and direction of locomotive in - event 4. |
| Event 4 acting time - hour | 137 | 0-23 | 0 | Set event 4 running time : hour. |
| Event 4 acting time - min | 138 | 0-59 | 0 | Set event 4 running time : min. |
| Event 4 acting time - sec | 139 | 0-59 | 0 | Set event 4 running time : sec. |

| CV Name | CV | Range | Default | Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Event Control | 140 | - | 0 | Decimal Value |  |  | 0 No Event |  |  |  |  |  |  |
|  |  |  |  | 1 |  |  | Event 1. |  |  |  |  |  |  |
|  |  |  |  | 2 |  |  | Event 1+Event 2. |  |  |  |  |  |  |
|  |  |  |  | 3 |  |  | Event 1+Event 2+Event 3. |  |  |  |  |  |  |
|  |  |  |  | 4 |  |  | Event 1+Event 2+Event 3+Event4. |  |  |  |  |  |  |
|  |  |  |  | 10 |  |  | Repeating Event 1+Event 2. |  |  |  |  |  |  |
|  |  |  |  | 11 |  |  | Repeating Event 1+Event 2+Event 3. |  |  |  |  |  |  |
|  |  |  |  | 12 |  |  | Repeating Event 1+Event 2. +Event3+Event4. |  |  |  |  |  |  |
| Current Fuel/Coal | 142 | 0-255 | - | Equivalent to CV894, Only Readable/writeable in Operational Level Mode. |  |  |  |  |  |  |  |  |  |
| Current Water | 143 | 0-255 | - | Equivalent to CV895, Only Readable/writeable in Operational Level Mode. |  |  |  |  |  |  |  |  |  |
| Initial Coal | 144 | 0-254 | 254 | The initial value of CV142/CV894 upon power up cycle. |  |  |  |  |  |  |  |  |  |
| Initial Water | 145 | 0-254 | 254 | The initial value of CV143CV895 upon power up cycle. |  |  |  |  |  |  |  |  |  |
| Simulation Control Flag | 146 | 0-3 | 0 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | B7 | B6 | B5 |  | B4 |  | B3 | B2 | B1 | B0 |
|  |  |  |  | X | X | X |  | X |  | X | X | • | • |
|  |  |  |  | B0: Simulation On/Off |  |  |  |  | 1-Simulation On. 0- Simulation Off. |  |  |  |  |
|  |  |  |  | B1: Light Pattern 1 |  |  |  |  | 1-Light Pattern 1 activated upon running out of water/coal. 0- No lighting effect upon running out of water/coal. |  |  |  |  |
| Motor Control algorithm | 150 | 0-1 | 0 | Decimal Number: 0: PID algorithm 0 (used in IntrainV1.3). 1: PID algorithm 1 (include the control for Lima Locomotive). |  |  |  |  |  |  |  |  |  |

| CV Name | CV | Range | Default | Description |
| --- | --- | --- | --- | --- |
| Fuel/Coal | 894 | 0-255 | - | Amount of virtual fuel/coal stored in the locomotive for driving simulation. Only readable/writeable in Operational Mode. |
| Water | 895 | 0-255 | - | Amount of virtual water stored in the locomotive for driving simulation. Only readable/writeable in Operational Mode. |

1.3 Hardware

| Function | Description |
| --- | --- |
| Main MPU | Provides the main functionality - decodes track based signal, supplies PWM output to motor and provides functional output. The unit will support back EMF feedback control. |
| Amp | Amplify PWM signal to drive the motor. Capable of driving 1.0A continuous, 1.5A peak and stall. |
| Receive electronics | Receives the communications signal from the track and converts it to be compatible with the MPU. Also enables the module to operate under standard DC and pick-up asymmetric DCC. |
| Transmit electronics | Transmits the RailCom® signal to the track. |

2 Regulations

The systems is built to the following safety and EMC standards:

EN71 Part 1 Mechanical and Physical properties.

EN71 Part 2 Flammability.

EN71 Part 3 Specification for the migration of certain elements.

EN71 Part 5 Chemical Toys (sets) other than experimental sets.

EN71 Part 6 Graphic Symbol for age warning abeling.

EN50088 Safety of Electrical Toys (both battery and mains).

EN61558-2-7 Specification for Low Voltage Power Supply Units - toys. (Previously EN60742).

EN55014 EMC The Electro Magnetic Compatibility Directive.

R&TTE (The Radio Equipment and Telecommunications Terminal Equipment Directive).

Australia/New Zealand 3108:1994. Approval and Test Specification-particular requirements for isolating transformers and safety

isolating transformers.

ASTM (USA) F963A Mechanical and Electrical Toy Safety (ASTM-American Society of Testing and Materials).

CPSC (USA) (Consumer Product Safety Commission).

FCC (USA version of the EN55014 EMC).

UL Regulations: including UL696 and UL697 for the PSU - UL817 SPT-2 for cable assembly.

CUL Regulations: CAN/CSA-C22, No 173-M1983.

For due diligence reasons, the system is designed to pass EN55022 - general EMC.

For more information visit :www.hornby.com