Power Cab

System Reference

Manual

Rev. 1.65

1

12:00AM

L1-3---

Power Supply

P114

LOC:1234

FWD: 000

NCE CAB

Optional Extra

Cab

Power Panel

Red LED on

bottom of panel

6 Wire FLAT

Cable

Basic Set Up of the

Power Cab System

Your Power CabTM is a full featured DCC system. It must be used with the 6 wire,

FLAT cable supplied. All features described in this manual are available to you when

used in the Power CabTM mode.

If you own or operate with a PowerHouseTM or Power ProTM NCE system, you can

use the Power CabTM    as a Pro CabTM using the 4 wire coiled cord that is also supplied.

BE CAREFUL

An NCE P114 power supply (13.5VDC Regulated) is the supplied transformer for

your system.

2

Power Panel (marked PCAB-PP)

Coil Cord (4 wire) to use as a Pro Cab

Power CabTM system reference manual

FWD: 000    -------

LOC: 003    00:03AM

Quick Start

Contents of the Power CabTM System:

Power CabTM

Seven foot flat cable

Power Supply

We recommend that you follow the instructions below to connect your system to a

small test track first. After your system is checked out completely and known to be

working then consider the final installation locations for components and connections

to the track.

TRYING OUT YOUR POWER CABTM:

#1 Assemble the Power Panel by screwing the black plate to the circuit board with

screws provided.

#2 Connect the long 7 foot flat cable from the Power CabTM into the socket marked

(on the drawing) POWER CABTM (left socket).

#3 Use a small, flat screw driver to attach the test track feeder wires to the small

two place connector. Make sure that there are no shorts or frayed wires between

the connections.

#4 If you have a locomotive with a DCC decoder already installed place it on the

track.

#5 Plug the power supply into the socket marked POWER. Plug the power supply

into a wall outlet.

#6 The display of the cab should look something like:

LOC: 003    00:03AM

FWD: 000    -------

This is what we call the ‘NORMAL DISPLAY’. The fast clock in the upper right hand

corner will most likely show a different time.

The following cab buttons must be pressed in the proper sequence to acquire control

of the locomotive #3 (in the above example locomotive #3 is already selected):

Note: By convention, locomotive decoders usually have their short address set to 3

at the factory.

a. Press SELECT LOCO button once.

b. Press the “3” button once. DO NOT press 0 then 3.

c. Press ENTER.

TIP #1 If you make a mistake while entering numbers on the cab just keep pressing

more numbers until the number entry field clears, then enter the correct numbers.

TIP #2 Whenever the cab is waiting for you to enter data (a flashing black square is

on the screen at the point where the numbers will go) you can press the PROG/ESC

key to “escape” what you were doing and return to normal operation mode.

TIP #3 Whenever you have pressed a button (such as SELECT LOCO) and you

want to keep what ever number that is already on the screen just press ENTER.

#7 To operate the locomotive, the following controls are used:

a. The thumbwheel will increase/decrease speed.    The speed control buttons

that flank the thumbwheel on either side may also be used as desired to

control the loco speed.

b. Pressing the DIRECTION button will cause the loco direction to reverse.

CONGRATULATIONS! You are now operating a locomotive with DCC control. In

most cases, less than ten minutes has elapsed since you started reading this Quick

Start Guide.

3

This Document has been created with a trial version of deskUNPDF.    To

purchase a copy of deskUNPDF and remove the trial restrictions, please visit

www.docudesk.com or email sales@docudesk.com for assistance.